BITS PILANI, DUBAI CAMPUS

DUBAI INTERNATIONAL ACADEMIC CITY, DUBAI

FIRST SEMESTER 2023 – 2024

COURSE: CSF303 (Computer Network)

COMPONENT: Tutorial Sheet 5			DATE: 11th March 2024

- A message 1001 1100 1010 0011 is transmitted using the Internet Checksum (using 4-bit words). What is the value of the checksum?

1001

1100

1010

0011

0000

Sum = 100010

Wrapped Sum = 0010 + 10 = 0100

Checksum = 1011

- Calculate possible values of back off time TB for different values of no. of attempt K when stations on an ALOHA network are a maximum of 600 km apart and the signals are propagate on the channel at the speed of 3 X 10^8 m/s.

- A pure ALOHA network transmits 200-bit frames on a shared channel of 200 kbps. What is the requirement to make this frame collision-free?

- A pure ALOHA network transmits 200-bit frames on a shared channel of 200 kbps. What is the throughput if the system (all stations together) produces?

  - 1000 frames per second

  - 500 frames per second

  - 250 frames per second

- A Slotted ALOHA network transmits 200-bit frames on a shared channel of 200 kbps. What is the throughput if the system (all stations together) produces?

  - 1000 frames per second

  - 500 frames per second

  - 250 frames per second