BITS PILANI, DUBAI CAMPUS

DUBAI INTERNATIONAL ACADEMIC CITY, DUBAI

SECOND SEMESTER 2023 – 2024

COURSE: CSF303 (Computer Network)

COMPONENT: Tutorial Sheet 6			DATE: 18th March 2024

- A CSMA/CD network has a bandwidth of 10 Mbps. If the maximum propagation time is 25.6 μs, what is the minimum size of the frame?

- A CSMA/CD network has a bandwidth of 1Mbps. If the maximum propagation time is 1 msec, what is the minimum size of the frame?

- Consider CSMA/CD. Let B=t/D, where t is the (ave.) propagation delay, and D (ave.) transmission delay. The throughput is E=1/[1+5B].

Given:

Transmission rate C=100Mbps,

Frame length L=2000 bits,

Cable distance d=1 mile (1 mile = 1.60934 km) ,

and assume that signals travel at the speed of light in the cable.

  - Find t and D.

  - Find B and E.

  - If C=1Gbps, repeat (a) and (b).

  - When the transmission rate increases, does E increase or decrease? Why?

- What is the length of a contention slot in CSMA/CD for:

  - 2-km cable (signal propagation speed is 82% of the speed in vacuum)?

  - 40-km multimode fiber optic cable (speed is 65% speed in vacuum)?

a. Propagation speed in twin lead=2.46∗10^8m/s

     Signal propagation speed=2.46∗10^ 8 m/s

     d=2000 m

     Tp=2000/2.46∗10^8 sec

     Contention Period=2Tp=2∗(2000/02.46∗10^8 ) sec sec

b. Propagation speed in multimode fiber=1.95∗10^8m/s

     Signal Propagation speed=1.95∗10^8m/s

     Contention Period=2∗(40000/1.95∗10^8 )sec

- Suppose two nodes, A and B, are attached to opposite ends of an 1200m cable, and that they each have one frame of 1,500 bits (including all headers and preambles) to send to each other. Both nodes attempt to transmit at time t=0. Suppose there are four repeaters between A and B, each inserting a 10 microseconds processing delay. Assume the transmission rate is 100 Mbps, and CSMA/CD with backoff intervals of multiples of 135 microseconds delay is used. After the collision, A draws K=0 and B draws K=1 in the exponential backoff protocol. Ignore the jam and the 96-bit time delay.

  - What is the one-way propagation delay (including repeater delays) between A and B in seconds? Assume the signal propagation speed is 2*10^8m/sec.

  - At what time (in seconds) is A’s packet completely delivered at B?

  - At what time (in seconds) is B’s packet completely delivered at A?

- Suppose nodes A and B are on same 10 Mbps Ethernet segment and the propagation delay between two nodes is 225 bit times. Suppose A and B send frames at t=0, the frames collide then at what time, they finish transmitting a jam signal. Assume a 48 bit jam signal

- Suppose nodes A and B are attached to opposite ends of the cable with propagation delay of 12.5 ms. Both nodes attempt to transmit at t=0. Frames collide and after first collision, A draws k=0 and B draws k=1 in the exponential back off protocol. Ignore the jam signal. At what time (in seconds), is A’s packet completely delivered at B if bandwidth of the link is 10 Mbps and packet size is 1000 bits.